      
                                                                   [image: image3.png]KNOWLEDGE & INPUT

AG R@LAN





[image: image1.jpg]WATERMARK METER - 30 KTCD-NL
(Green Case)

Hook the meter's leads to the sensor's wires with the alligator clips, being
sure the separate leads are not touching each other.

Press *READ’ to wake up the meter, you will see “- -* in the display. The
meter will stay awake for 5 seconds (to keep meter awake for 60 seconds,
press “TEMP” before “- -* goes away).

Press “READ" again while *- = is in the display. The soil moisture reading
will immediately appear in the display and remain there for 60 seconds while
you record it. The meter wil then turn itseif off.

When taking readings, the soil temperature adjustment should be set for
early morning ambient temperature. This setting compensates for seasonal
variations in soil temperature, which can go from the sixties in the spring to
the eighties in the summer. This variation in soil temperature can affect the
readings by 1% per degree Fahrenheit, so the temperature compensation
greatly improves the accuracy of your readings

To check the temperature settings, press “TEMP”. The temperature setting
and the scale (°F or °C) will alternate in the display.



[image: image2.jpg]To change the temperature scale, press and hold “READ", then alternately press
“TEMP" until the desired scale (°F or °C) appears in the display and then release
READ button

To change the temperature setting, press and hold “TEMP" then press *READ" to
change the setting. The temperature setting will begin to increase until the desired
setting appears in the display. The full scale of temperature setting is 41°F (5°C) to
105°F (40°C). Once the temperature scrolls up to 105°F, it will go to 41°F and begin
scrolling upwards again. You can reverse the direction of scrolling at any time by
releasing the "READ" button and depressing it again (while continuing to hold
“TEMP" down),

The temperature settings you programmed in will remain until you change them. The
meter comes with a default setting of 75°F

The meter has a built in test function. To test the meter for accuracy, with the
temperature sefting at 75°F (24°C), press and hold "READ" and “TEST"
simultaneously. A reading of between 95 and 105 should appear in the display. This
reading indicates the meter is functioning properly. During test, make sure cable
leads are not touching or hooked to a sensor.

This digital meter has a full range of 0 to 199 centibars buit in

The digital meter utiizes solid state electronics and is sensitive to extreme heat. Do
not store the meter on the dashboard or any other very hot location. Replace with a
good quality 9V alkaline battery at least once each year. The meter has a low battery
indicator and the battery should be replaced whenever “LO" appears in the display.




AGROLAN   KnowLedge  & Input    Moshav  12921   ISRAEL  Email:agrolan@agrolan.co.il       Tel  972-4-6763091     Fax- 972-4-6763093    http://www.agrolan.co.il

[image: image3.png]_911248119.doc
�



�
















